Stability in Vehicular Ad Hoc Networks Using Adaptive Gossip-Based Dissemination

by

Boris Gor: 24262782

Supervisor:

Professor Adnan Al-Anbuky

A Research Project Report submitted to Auckland University of Technology in partial fulfilment of the requirements for the degree of Master of Engineering

Faculty of Design and Creative Technologies School of Engineering, Computer and Mathematical Sciences

2026 (v0.0.5)

Abstract	3

1.	Introduction	4

2.	Literature Review	6

3.	System Model and Design	10

4.	Implementation	12

5.	Experiments and Results	16

5.1	Experimental Setup	16

5.2	Performance Metrics	16

5.3	Delivery Ratio	16

5.4	End-to-End Latency	16

5.5	Transmission Overhead	16

5.6	Emergency Convergence Time	16

5.7	Scenario-Based Evaluation	16

5.8	Comparative Results	16

6.	Discussion	17

7.	Conclusion and Future Work	18

References	19

# Abstract

Vehicular Ad Hoc Networks (VANETs) enable direct vehicle-to-vehicle communication for safety-critical and traffic efficiency applications, however, their highly dynamic topology and frequent network fragmentation make stable message dissemination difficult to achieve. Routing-based approaches degrade rapidly under high mobility, while infrastructure-heavy or computationally intensive solutions introduce practical deployment constraints that limit their applicability in realistic sparse environments.

This research project develops and evaluates an adaptive gossip-based (epidemic) dissemination protocol designed to maintain stable and predictable operation under prolonged network fragmentation. The proposed approach employs probabilistic forwarding with bounded redundancy and adaptive control, allowing messages to propagate without reliance on persistent routes or centralized coordination. Rather than targeting optimal delivery performance, the design prioritises predictable, resource-aware behaviour under intermittent connectivity, treating temporary non-delivery as a normal operating condition rather than a failure state.

The protocol is implemented using the Contiki-NG operating system, targeting resource-constrained embedded platforms, and is evaluated primarily through simulation-based experiments in the Cooja environment across a range of mobility, density, and fragmentation scenarios. In situations where no neighbouring vehicle or Roadside Unit (RSU) is reachable within radio range, messages are retained locally using a bounded buffer and carried opportunistically until contact is re-established or storage pressure necessitates displacement of older data. This store-carry-forward behaviour is integrated directly into the protocol design.

Comparative evaluation against baseline dissemination strategies, including flooding and a non-adaptive gossip variant, focuses on behavioural stability, communication overhead, and graceful degradation under fragmentation. The evaluation provides insight into the behaviour and inherent limitations of stability-oriented gossip dissemination in sparse vehicular networks, supporting the feasibility of lightweight, infrastructure-independent communication mechanisms in environments where connectivity cannot be assumed.

# Introduction

Vehicular Ad Hoc Networks (VANETs) enable direct wireless communication between vehicles and surrounding infrastructure to support safety-critical and traffic efficiency applications such as emergency notifications, collision avoidance, and cooperative traffic management. Unlike conventional wireless networks, VANETs are characterised by high node mobility, rapidly changing network topology, variable node density, and frequent network fragmentation. These properties significantly complicate reliable message dissemination, particularly in sparse or intermittently connected environments.

Much existing VANET research prioritises delivery-centric performance metrics, often under assumptions of sufficient connectivity or infrastructure support. However, in many realistic vehicular scenarios—such as highways, rural areas, and regions with limited Roadside Unit (RSU) coverage—persistent end-to-end connectivity cannot be assumed. Vehicles may experience extended periods of isolation, during which no forwarding opportunities are available. In such conditions, dissemination mechanisms that rely on stable routes or continuous connectivity often exhibit unpredictable behaviour, excessive control overhead, or inefficient resource usage.

In this work, communication stability is understood as the ability of a dissemination mechanism to operate in a predictable and resource-bounded manner under highly dynamic conditions, including intermittent connectivity, sustained network fragmentation, and the absence of end-to-end communication paths. Temporary non-delivery is not treated as a failure; instead, stable operation is characterised by bounded communication overhead, controlled use of local storage, and gradual, non-oscillatory degradation of behaviour as network conditions worsen.

A key distinction addressed in this project is the difference between transient disconnections and prolonged fragmentation. While brief connectivity interruptions are commonly considered in existing dissemination designs, prolonged fragmentation introduces qualitatively different challenges. When the duration of isolation exceeds the inter-arrival time of generated messages, dissemination behaviour becomes dominated by storage pressure and buffer management rather than forwarding efficiency. Protocols that implicitly assume eventual contact or effectively unbounded storage may therefore exhibit uncontrolled resource consumption or silent information loss under such conditions.

Decentralised gossip-based (epidemic) dissemination mechanisms provide an alternative to route-dependent communication by exploiting opportunistic forwarding and controlled redundancy. By relying on local interactions and probabilistic forwarding, such approaches can tolerate frequent topology changes and uncertain connectivity. However, epidemic dissemination is sensitive to network density, forwarding parameters, and buffer availability, making static configurations unsuitable across the diverse operating conditions encountered in realistic vehicular networks. This motivates the use of adaptive mechanisms combined with explicit resource bounding.

The objective of this research project is to investigate whether an adaptive gossip-based (epidemic) dissemination protocol can provide stable and predictable communication behaviour in vehicular ad hoc networks under prolonged fragmentation, while remaining lightweight and suitable for resource-constrained embedded platforms. The focus is not on maximising delivery ratio or minimising latency under ideal conditions, but on understanding dissemination behaviour when connectivity is intermittent and resources are finite.

To address this objective, the proposed approach is implemented using the Contiki-NG operating system and evaluated primarily through simulation-based experiments conducted in the Cooja environment across a range of mobility, density, and fragmentation scenarios. Performance analysis emphasises behavioural stability, communication overhead, and bounded buffer usage under fragmentation, rather than delivery-centric metrics alone.

Contributions

The main contributions of this research project are as follows:

- The design and implementation of an adaptive gossip-based (epidemic) dissemination protocol targeting stable operation under prolonged network fragmentation.

- An experimental, simulation-based evaluation of dissemination behaviour and degradation patterns under sparse and intermittently connected vehicular scenarios.

- An analysis of bounded resource behaviour, focusing on controlled buffer usage and predictable message lifetime under sustained data generation.

- A comparative evaluation against baseline dissemination strategies, including flooding and a non-adaptive gossip variant.

- A proof-of-concept execution of the proposed protocol on real embedded hardware platforms to demonstrate practical feasibility.

The remainder of this report is organised as follows. Chapter 2 reviews related work on VANET dissemination, fragmentation tolerance, gossip-based approaches, and resource constraints. Chapter 3 describes the system model and design assumptions underlying the proposed protocol. Chapter 4 outlines the implementation details, followed by experimental evaluation in Chapter 5. Chapter 6 discusses observed behaviour, limitations, and trade-offs, and Chapter 7 concludes the report and outlines directions for future work.

# Literature Review

## Communication Challenges in Vehicular Ad Hoc Networks

Vehicular Ad Hoc Networks (VANETs) operate under conditions fundamentally different from those assumed in conventional wireless networks. High vehicle mobility, rapidly changing topology, variable node density, and frequent network fragmentation significantly affect communication reliability and make persistent end-to-end connectivity difficult to achieve [1]. In contrast to traditional networks, link availability in VANETs is highly transient and strongly dependent on traffic density and mobility patterns.

Empirical studies based on real mobility traces indicate that extended periods of disconnection are a natural characteristic of mobile systems rather than an exceptional condition. Chaintreau et al. analyse multiple real-world datasets and show that inter-contact time distributions exhibit heavy-tailed behaviour, resulting in long intervals without contact opportunities between nodes [2]. These findings imply that frequent or regular encounters cannot be assumed even in environments with continued node mobility.

Analytical modelling of vehicular connectivity supports this observation. Khabazian and Ali demonstrate that vehicular networks do not maintain full communication connectivity at all times and may fragment into multiple disjoint clusters when inter-vehicle spacing exceeds transmission range [3]. Their model explicitly characterises the durations of connectivity availability and unavailability, showing that extended connectivity gaps are an expected property of sparse vehicular scenarios.

When the duration of isolation exceeds the inter-arrival time of generated messages, dissemination behaviour becomes increasingly dominated by storage pressure and buffer management rather than forwarding efficiency alone. This phenomenon is a central concern in Delay-Tolerant Networking (DTN) research, which explicitly models message retention and delivery under prolonged disconnection and limited resources [11]. These observations motivate dissemination strategies that treat prolonged fragmentation as a normal operating condition rather than a failure state.

## Routing-Based and Infrastructure-Dependent Dissemination Approaches

Early VANET dissemination research largely relies on routing protocols adapted from Mobile Ad Hoc Networks (MANETs). These protocols aim to establish and maintain multi-hop paths and typically optimise delivery-centric metrics such as packet delivery ratio and end-to-end delay. Surveys of VANET routing protocols consistently show that routing performance degrades rapidly under high mobility and frequent topology changes [4].

Infrastructure-assisted dissemination approaches attempt to mitigate routing instability through the use of Roadside Units (RSUs), cellular connectivity, or central coordination mechanisms [5]. While such approaches can improve performance in dense urban deployments, they inherently assume the availability of supporting infrastructure. In highway, rural, or sparsely instrumented environments, these assumptions frequently do not hold, limiting the applicability of infrastructure-dependent solutions.

Consequently, both routing-based and infrastructure-assisted approaches often exhibit unstable behaviour or excessive overhead in scenarios characterised by sustained fragmentation and intermittent connectivity.

## Broadcast and Flooding-Based Dissemination

Broadcast-based dissemination avoids explicit route establishment by allowing nodes to rebroadcast received messages. Flooding represents the simplest form of broadcast dissemination and offers high reachability under uncertain connectivity. However, uncontrolled flooding results in substantial redundancy and channel contention.

This effect, known as the broadcast storm problem, has been formally analysed by Tseng et al., who show that redundant rebroadcasts lead to severe contention, collisions, and performance degradation in mobile ad hoc networks [6]. Various suppression-based and probabilistic broadcast techniques have been proposed to mitigate this problem, but their effectiveness remains sensitive to node density and configuration parameters.

Flooding-based approaches therefore primarily serve as baseline mechanisms, illustrating the trade-off between reachability and communication overhead rather than providing a scalable solution.

## Gossip- and Epidemic-Based Dissemination

Gossip-based, or epidemic, dissemination propagates information through opportunistic and probabilistic exchanges between nodes without requiring persistent routes or global coordination. The paradigm originates from epidemic algorithms developed for replicated database maintenance [7].

Epidemic dissemination has been shown to tolerate frequent topology changes and intermittent connectivity more effectively than routing-based approaches in mobile and vehicular environments [8]. However, fundamental analysis of gossip-based routing demonstrates that static forwarding probabilities perform well only within limited operating regimes. Haas, Halpern, and Li show that inappropriate parameter selection can lead either to excessive redundancy or to poor delivery performance, depending on network conditions [15].

Adaptive epidemic dissemination schemes attempt to address this sensitivity by adjusting forwarding behaviour based on observed network conditions. For example, Nizamoglu et al. propose a density-adaptive epidemic mechanism that exploits lane structure in vehicular networks to improve dissemination efficiency under varying traffic densities [9]. While such approaches improve delivery-overhead trade-offs, they primarily optimise delivery performance rather than explicitly targeting stable behaviour under prolonged fragmentation and bounded resource constraints.

## Trickle-Based and Timer-Controlled Dissemination

Timer-controlled dissemination mechanisms regulate transmission frequency using adaptive suppression rules. The Trickle algorithm dynamically reduces redundant transmissions once information consistency is achieved [10].

Trickle relies on receiving transmissions from neighbouring nodes to trigger suppression. During extended isolation, these conditions are not met, causing transmission intervals to increase toward their maximum values [10]. Consequently, newly generated information may propagate slowly after connectivity is re-established, limiting responsiveness under prolonged fragmentation.

Although effective at reducing overhead under moderate connectivity, Trickle’s suppression-based behaviour is less suited to environments where sustained disconnection is common.

## Store-Carry-Forward and Resource Constraints

Store-carry-forward dissemination originates from DTN architecture, which explicitly supports intermittent connectivity by allowing nodes to buffer messages and exploit mobility for delivery [11].

DTN research shows that buffer management decisions strongly influence dissemination behaviour under prolonged disconnection. Krifa et al. demonstrate that message drop policies directly affect delivery ratio, stability, and overhead in epidemic routing scenarios [12]. Surveys of DTN buffer management further highlight that many existing protocols assume large buffers or prioritise delivery probability over predictable resource usage [13].

These findings indicate that epidemic dissemination in fragmented networks requires explicit resource bounding and controlled buffer behaviour to achieve stable and predictable operation.

## Evaluation Perspective and Research Positioning

Most VANET dissemination studies evaluate protocols using delivery-centric metrics such as packet delivery ratio and latency. DTN surveys argue that these metrics alone are insufficient in environments characterised by intermittent connectivity and prolonged fragmentation [14]. In such conditions, behavioural stability, resource usage, and degradation patterns become equally important evaluation dimensions.

Accordingly, this work positions itself as an investigation into stable, predictable, and resource-bounded gossip-based dissemination, rather than optimal delivery under ideal connectivity assumptions.

Table 2.1: Dissemination Paradigms and Design Properties

| Approach | Connectivity Assumption | Tolerates Prolonged Fragmentation | Overhead Bounded | Predictable Behaviour |
| --- | --- | --- | --- | --- |
| Routing-based | Persistent paths | Low | Low | Low |
| Infrastructure-assisted | Dense RSU coverage | Medium | Medium | Medium |
| Flooding | None | High | No | Low |
| Gossip / Epidemic | Opportunistic contacts | High | Medium | Medium |
| Trickle-based | Periodic neighbour interaction | Medium | High | Medium |
| This work | Intermittent and prolonged disconnection | High | Yes | High |

# System Model and Design

## System overview

The considered system consists of two types of nodes: mobile nodes, representing vehicles, and Roadside Units (RSUs), which provide intermittent access to fixed infrastructure. Mobile nodes move autonomously and communicate opportunistically with neighbouring mobiles using short-range wireless broadcast. RSUs act as collection points that receive data from mobile nodes whenever connectivity is available.

Figure 3.1 illustrates the system model, where mobile vehicles exchange DATA and EMERGENCY messages using gossip-based broadcast and opportunistically flush buffered data to an RSU when connectivity becomes available.

Figure 3.1: Illustration of gossip-based dissemination among vehicles and opportunistic buffer flush from a mobile node to an RSU.

The system is designed for environments where end-to-end connectivity cannot be assumed and periods of disconnection are a normal operating condition rather than an exception. Mobile nodes therefore follow a store-carry-forward paradigm: data is generated locally, disseminated opportunistically between mobiles, and eventually delivered to an RSU when one becomes reachable. Communication with backend services or cloud infrastructure is conceptually possible through RSUs but is explicitly outside the scope of this work.

## Node Types and Roles

### Mobile Nodes

Mobile nodes represent vehicles participating in the network. Each mobile node performs the following functions:

- periodic generation of application data messages;

- generation of emergency messages upon detection of sudden stopping events;

- broadcast-based dissemination of data using a gossip-style mechanism;

- buffering of locally generated and relayed messages in a fixed-size local store;

- opportunistic forwarding of buffered data to RSUs when an RSU becomes reachable.

Mobile nodes are resource-constrained and execute the protocol on embedded hardware consistent with Contiki-NG deployment targets.

### Roadside Units (RSUs)

RSUs represent fixed infrastructure nodes with wireless connectivity to passing mobiles. Their role is intentionally minimal:

- receive DATA and EMERGENCY messages from mobile nodes;

- acknowledge received messages using unicast acknowledgements;

- propagate active EMERGENCY state via RSU acknowledgements, with a single local re-broadcast;

- signal successful data reception so that mobile nodes can safely remove stored messages.

RSUs do not aggregate data, perform analytics, or persist data to a backend database in this study. This design choice simplifies the system and focuses the evaluation on dissemination and delivery behaviour rather than backend processing.

# Implementation

## Implementation Overview

This chapter describes the concrete implementation of the proposed dissemination system introduced in Chapter 3. The protocol is implemented in C on top of the Contiki-NG operating system and targets constrained embedded devices compatible with IEEE 802.15.4 and 6LoWPAN networking.

Two executable node roles are implemented as separate programs:

- Mobile node implementation, responsible for data generation, gossip-based dissemination, local buffering, emergency detection, and opportunistic delivery to RSUs.

- RSU implementation, responsible for receiving mobile data, issuing acknowledgements, and limited emergency message re-broadcasting.

A shared protocol specification, including message formats and utility functions, is defined in a common header to ensure consistency across node types.

The implementation prioritises clarity, determinism, and bounded resource usage over maximum throughput or protocol optimality, in line with the design goals discussed in Chapter 3.

## Software Structure

The implementation is organised into three main source files:

- bgkp_protocol.h: defines the wire format, message structures, helper functions, and shared protocol constants.

- mbl.c: implements the mobile node logic, including gossip dissemination, peer selection, carry buffer management, and RSU flushing.

- rsu.c: implements the RSU behaviour as a minimal data sink with acknowledgement support.

This separation allows protocol definitions to remain independent of node-specific logic and simplifies further extension or experimentation.

## Message Types and Wire Format

All messages exchanged between nodes follow a common BGKP frame structure transported over UDP. Each frame contains:

- a protocol marker and length field;

- sender and origin identifiers;

- message type and message identifier;

- optional payload;

- a timestamp used for logging and ordering.

Message Types

The implementation supports the following message types:

- DATA: periodically generated application messages containing mobility-related metrics.

- EMERGENCY: event-driven messages triggered by sudden stop detection.

- QUERY: periodic broadcast messages used to announce presence and exchange limited state.

- ACK: acknowledgement messages used to confirm reception.

Multi-hop forwarding preserves the original creator’s metadata, while the physical sender identifier is updated at each hop. Lightweight checksum validation is used for frame integrity checking. The BGKP wire format is summarised in Table 4.1.

Table 4.1: BGKP Frame Structure

| Field | Size(Bytes) | Description |
| --- | --- | --- |
| Marker | 4 | Protocol identifier ("BGKP") |
| Length | 3 | Frame length |
| Sender ID | 4 | Physical sender identifier |
| Message Type | 1 | DATA, EMERGENCY, QUERY, or ACK |
| Message ID | 4 | Unique message identifier |
| Target ID | 4 | Intended destination or broadcast |
| Origin ID | 4 | Original message creator |
| Payload | Variable | Message‑specific payload |
| Timestamp | 4 | Creation time (ms) |
| Checksum | 1 | Simple frame checksum |

The different payloads are described in Table 4.2 below.

Table 4.2: BGKP Payload Types

| Payload Type | Fields (summary) | Purpose |
| --- | --- | --- |
| DATA | Position, speed, direction, TTL, peers | Periodic mobility data dissemination |
| EMERGENCY | Event code, position, speed, direction | Sudden-stop event notification |
| QUERY | Node identifier, emergency flag (summary) | Presence announcement and state exchange |
| ACK | Acknowledged message metadata | Reception confirmation |

## Mobile Node Implementation

### Mobility Input and Event Detection

Mobile nodes obtain position and timing information through a UART-based interface that emulates a GPS-like sensor. In simulation this data is injected by the Cooja mobility script, while on hardware platforms the same interface can be connected to an external positioning source. Position updates are processed to estimate displacement, speed, and direction without relying on floating-point arithmetic.

An emergency braking event is detected when node displacement remains below a fixed threshold for a minimum duration. When such a condition is met, an EMERGENCY message is generated. A corresponding termination message is generated once normal movement resumes.

### Gossip Dissemination

Mobile-to-mobile dissemination is implemented as a controlled gossip mechanism using local broadcast.

Each DATA message originated by a mobile node is transmitted during a bounded retry window with the retry count and inter-transmission jitter defined as fixed parameters. The specific values used in the experiments are listed in Chapter 5. Rather than rebroadcasting indiscriminately, the node maintains a small set of preferred peers, referred to as the preferred peer set, which influences acknowledgement handling and retransmission behaviour.

Gossip retransmissions are scheduled using Contiki timers with small random jitter to reduce repeated collisions and synchronisation effects.

### Preferred Peer Selection

Each mobile node maintains a fixed-size preferred peer set containing peer identifiers. The set is intentionally limited to at most two entries in order to bound state size and control acknowledgement overhead on resource‑constrained embedded platforms, while still allowing basic redundancy and temporal diversity in forwarding. The preferred peer set is used as a lightweight control structure to limit gossip overhead and stabilise dissemination behaviour.

Peers are dynamically promoted into the preferred peer set based on observed acknowledgements during DATA transmission windows. Conversely, a peer is removed from the list only after a configurable number of consecutive missed acknowledgement cycles.

This mechanism is intentionally simple and avoids explicit neighbourhood discovery, global state exchange, or density estimation. It realises the adaptive peer selection concept described in Section 3.5.

### Carry Buffer and Eviction Policy

To tolerate prolonged disconnection, each mobile node maintains a local carry buffer that stores both locally generated and relayed messages. Messages are stored in a compact record format rather than as full wire frames to reduce memory consumption.

When the carry buffer becomes full, the oldest stored message is:

- broadcast once using the gossip mechanism as a best-effort delivery attempt;

- immediately evicted to free space for new messages.

This policy ensures bounded memory usage and graceful degradation under sustained data generation. The carry buffer implements the store–carry–forward model introduced in Chapter 3.

### Opportunistic RSU Flush

When a mobile node detects the presence of an RSU, buffered messages are flushed using unicast transmission. Each stored message is reconstructed into a full BGKP frame and sent directly to the RSU.

The carry buffer entry is removed only after an explicit acknowledgement is received from the RSU. This acknowledgement semantics provides a clear and reliable signal for successful delivery to infrastructure, distinct from acknowledgements used during gossip dissemination.

## RSU Implementation

The RSU implementation is intentionally minimal. An RSU:

- listens for incoming BGKP frames;

- validates and parses received messages;

- replies with acknowledgements for DATA and EMERGENCY messages;

- re-broadcasts each EMERGENCY message exactly once.

EMERGENCY messages are re-broadcast once by the RSU for immediate local awareness, while acknowledgement responses are used to inform newly arriving mobile nodes, avoiding sustained emergency flooding.

RSUs do not generate application data, perform aggregation, or interact with backend services in this implementation. This design choice isolates dissemination behaviour from backend-dependent effects and simplifies experimentation.

## Implementation Scope and Limitations

The implementation deliberately omits several aspects often addressed in production vehicular systems, including:

- cryptographic authentication or encryption;

- protection against malicious or misbehaving nodes;

- detailed modelling of physical-layer or MAC-layer effects;

- energy accounting or duty-cycle optimisation.

These aspects are considered orthogonal to the goals of this work and are left for future exploration.

# Experiments and Results

  - Experimental Setup

  - Performance Metrics

  - Delivery Ratio

  - End-to-End Latency

  - Transmission Overhead

  - Emergency Convergence Time

  - Scenario-Based Evaluation

  - Comparative Results

# Discussion

# Conclusion and Future Work

# References

[1] H. Hartenstein and K. Laberteaux, VANET: Vehicular Applications and Inter-Networking Technologies, Wiley, 2010.

[2] A. Chaintreau et al., “Impact of Human Mobility on Opportunistic Forwarding Algorithms,” IEEE Transactions on Mobile Computing, vol. 6, no. 6, 2007.

[3] M. Khabazian and M. Ali, “A Performance Modeling of Connectivity in Vehicular Ad Hoc Networks,” IEEE Transactions on Vehicular Technology, vol. 57, no. 4, 2008.

[4] S. Malik and P. K. Sahu, “A comparative study on routing protocols for VANETs,” Heliyon, vol. 5, no. 8, 2019.

[5] J. Santa, A. F. Gómez-Skarmeta, and M. Sánchez-Artigas, “Architecture and evaluation of a unified V2V and V2I communication system,” Computer Communications, 2008.

[6] Y.-C. Tseng et al., “The broadcast storm problem in a mobile ad hoc network,” Wireless Networks, vol. 8, no. 2, 2002.

[7] A. Demers et al., “Epidemic Algorithms for Replicated Database Maintenance,” ACM PODC, 1987.

[8] P. Spadaccino, F. Cuomo, and A. Baiocchi, “Epidemic and timer-based message dissemination in VANETs,” Electronics, vol. 9, no. 4, 2020.

[9] I. Nizamoglu, S. C. Ergen, and O. Ozkasap, “EpiDOL: Epidemic Density Adaptive Data Dissemination Exploiting Opposite Lane in VANETs,” EUNICE, 2013.

[10] P. Levis et al., “The Trickle Algorithm,” IETF RFC 6206, 2011.

[11] K. Fall, “A Delay-Tolerant Network Architecture for Challenged Internets,” ACM SIGCOMM, 2003.

[12] A. Krifa, C. Barakat, and T. Spyropoulos, “Message Drop and Scheduling in DTNs: Theory and Practice,” IEEE Transactions on Mobile Computing, vol. 11, no. 9, 2012.

[13] S. Rashid et al., “Buffer Management in Delay Tolerant Networks: A Survey,” EURASIP Journal on Wireless Communications and Networking, 2018.

[14] M. J. Khabbaz, C. M. Assi, and W. F. Fawaz, “Disruption-Tolerant Networking: A Comprehensive Survey,” IEEE Communications Surveys & Tutorials, vol. 14, no. 2, 2012.

[15] Z. J. Haas, J. Y. Halpern, and L. Li,

“Gossip-based ad hoc routing,”

IEEE/ACM Transactions on Networking, vol. 14, no. 3, 2006.