Stability in Vehicular Ad Hoc Networks Using Adaptive Gossip-Based Dissemination

by

Boris Gor: 24262782

Supervisor:

Professor Adnan Al-Anbuky

A Research Project Report submitted to Auckland University of Technology in partial fulfilment of the requirements for the degree of Master of Engineering

Faculty of Design and Creative Technologies School of Engineering, Computer and Mathematical Sciences

2026 (v0.0.1)

Abstract	3

1.	Introduction	4

2.	Literature Review	5

3.	System Model and Design	6

4.	Implementation	7

5.	Experiments and Results	8

5.1	Experimental Setup	8

5.2	Performance Metrics	8

5.3	Delivery Ratio	8

5.4	End-to-End Latency	8

5.5	Transmission Overhead	8

5.6	Emergency Convergence Time	8

5.7	Scenario-Based Evaluation	8

5.8	Comparative Results	8

6.	Discussion	9

7.	Conclusion and Future Work	10

References	11

# Abstract

Vehicular Ad Hoc Networks (VANETs) enable direct vehicle‑to‑vehicle communication for safety‑critical and traffic efficiency applications, yet their highly dynamic topology and frequent network fragmentation make stable message dissemination difficult to achieve. Routing‑based approaches often degrade under high mobility, while infrastructure‑heavy or computationally intensive solutions introduce practical constraints on deployment.

This research project investigates the use of adaptive gossip‑based (epidemic) dissemination as a lightweight and decentralized approach to improving communication stability in vehicular environments. The proposed approach employs probabilistic forwarding with bounded redundancy, allowing messages to propagate effectively without relying on persistent routes or centralized coordination. Particular emphasis is placed on the dissemination of emergency events, where timely and robust delivery is essential despite intermittent connectivity.

The protocol is implemented using the Contiki‑NG operating system and evaluated primarily through simulation‑based experiments in the Cooja environment under a range of mobility, density, and fragmentation scenarios. In addition, practical validation is performed on embedded hardware platforms to verify core protocol behavior and feasibility beyond simulation. Performance is examined through comparative evaluation against established baseline dissemination methods such as flooding and Trickle‑based approaches. The results provide practical insights into the design of lightweight, infrastructure‑independent communication mechanisms suitable for highly dynamic vehicular networks.

# Introduction

Vehicular Ad Hoc Networks (VANETs) enable direct communication among vehicles to support safety‑critical and traffic efficiency applications such as emergency braking alerts, collision avoidance, and cooperative traffic management. However, VANET environments are inherently challenging due to high vehicle mobility, rapidly changing network topology, and frequent network fragmentation, which significantly reduce the reliability of conventional message dissemination mechanisms.

Traditional routing protocols, originally designed for Mobile Ad Hoc Networks (MANETs), often degrade in vehicular scenarios as multi‑hop routes become invalid within short time intervals and control overhead increases under dynamic conditions. Infrastructure‑heavy solutions based on dense deployment of Roadside Units (RSUs) or centralized coordination can mitigate some of these issues, but introduce cost, deployment complexity, and limited scalability. Likewise, machine learning and AI‑based approaches frequently require computational resources and system complexity that are impractical for lightweight vehicular platforms.

Decentralized gossip‑based (epidemic) dissemination mechanisms offer an alternative approach that avoids reliance on stable routes or centralized control. By using probabilistic forwarding and bounded redundancy, gossip‑based protocols can tolerate frequent topology changes and temporary disconnections while avoiding excessive redundancy and broadcast storms caused by unrestricted message rebroadcasting. Adaptive variants of such protocols demonstrate that communication overhead can be regulated while maintaining acceptable delivery robustness in highly dynamic environments.

The objective of this research project is to investigate whether an adaptive gossip‑based dissemination strategy can improve communication stability in vehicular ad hoc networks while remaining lightweight, distributed, and suitable for embedded platforms. The proposed approach is implemented using the Contiki‑NG operating system and evaluated primarily through simulation‑based experiments in the Cooja environment under representative vehicular mobility scenarios. To complement the simulation results and verify practical feasibility, selected aspects of the protocol are also validated using real embedded hardware.

The remainder of this report is organized as follows. Chapter 2 reviews related work in VANET communication and epidemic dissemination strategies. Chapter 3 presents the system model and design of the proposed approach, followed by implementation details in Chapter 4.

Chapter 5 presents the experimental evaluation and results, Chapter 6 discusses the observed behavior and limitations, and Chapter 7 concludes the report and outlines directions for future work.

# Literature Review

# System Model and Design

# Implementation

# Experiments and Results

  - Experimental Setup

  - Performance Metrics

  - Delivery Ratio

  - End-to-End Latency

  - Transmission Overhead

  - Emergency Convergence Time

  - Scenario-Based Evaluation

  - Comparative Results

# Discussion

# Conclusion and Future Work

# References

[1]